Test report: MCS – Data rate limit

KIT WOM 5A MIMO G2 / WOM 5A MiMo+

- General informations

| Test Name | MCS – Data rate limit |
| --- | --- |
| Product Name | KIT WOM 5A MIMO G2 / WOM 5A MiMo+ |
| Product Code | 4301217 / 4750238 |
| PN Supplier | WB5nL3 / WB8nL3 |
| Tester | Gabriel Dias dos Santos - Reliability Analyst |
| Segment | Enterprise Networks |
| Date | Feb 25, 2026 |
| FW Version | APCPE.QM-1C.v7.8-Intelbras.20260204031117 |
| Result | [ ] Approved [X] Disapproved [ ] Inconclusive |

- Test Summary

| Objective | Validate the MCS – Data rate limit |
| --- | --- |
| Topology: | 2 Radios + 2 computers connected by LAN port + Iperf3 SW |
| Operating Mode: | Bridge |
| Frequency | 5G |
| Wi-Fi Protocol | n/a |
| Channel | 157 |
| Environment | Lab environment – RD |

- Execution & Results

| Test Case ID | Test Description | Expected Result | Actual Result | Status (Pass/Fail) |
| --- | --- | --- | --- | --- |
| TC-01 | Select MIMO 2X2 MCS8(30) in AP and CPE, and start the throughput test | The throughput should follow the data rate (MCS) | The throughput cannot follow the data rate (MCS) | FAIL |
| TC-02 | Select SISO 1X1 MCS0(15) in AP and CPE, and start the throughput test | The throughput should follow the data rate (MCS) | The throughput cannot follow the data rate (MCS) | FAIL |

- Observations & Technical Notes

The throughput cannot follow the data rate (MCS).

Observations

- If I defined SISO, the SNR and signal in AP is wrong:

- Data rate information is wrong in the SISO and MIMO mode in AP and CPE

- Conclusion

Throughput connection isn’t respecting the data rate of the MCS defined

- Photos

  - TC-01

  - TC-02

___________________________________

Gabriel Dias dos Santos - Reliability Analyst